IOWA DEPARTMENT OF NATURAL RESOURCES

ENVIRONMENTAL SERVICES DIVISION

Field Office 1

909 West Main, Manchester, Iowa 52057

Phone: (563) 927-2640          FAX: (563) 927-2075

REPORT OF INVESTIGATION

INVESTIGATION DATE                       	      CURRENT:  12/9/15

LAST: NA

TO:  John Foster, Agency Administrator, 925 Kerper Court, Dubuque, IA 52001

RE:  Notice of Violation    567—123.8(6) Structures.

Permit # Household Hazardous Materials RCC #31-SDP-07-96

PERSONS CONTACTED: Doug Hughes – Dubuque Metropolitan Sanitary Landfill

Kathleen Hennings – DNR, Des Moines

Distribution:  Field Office: Central Office:  Inspected Facility:

AUTHENTICATION

INSPECTOR:

Doug A. Hawker, Environmental Specialist Senior

REVIEWER:                                                                                                                                                   DATE:

Tom McCarthy, Environmental Specialist Senior

File Name:  31 SW DMSL 31SDP0796 120915 ins dah

OBSERVATIONS/RECOMMENDATIONS:

On December 9, 2015, I accompanied Kathleen Hennings from the Des Moines Central Office Solid Waste Section on an inspection of the Household Hazardous waste receiving and sorting building located at the Dubuque Metropolitan Sanitary Landfill.  We met with landfill manager, Doug Hughes.

Below is a list of observations made and comments received during the course of this inspection:

- DOT category signs posted on the wall designating separate areas for sorting

- A Gaylord of class 2.1 aerosols stored open on a pallet

- A pallet of Lead acid batteries stored on a pallet

  - Secondary containment had standing liquid that appeared to be oil

- Fluorescent bulbs stored in improperly marked containers

- Older, unsealed concrete with large visible cracks throughout

After confirming with 2 staff members it was confirmed that the SE corner of the building between the shelving and the DOT signs is the area is used for receiving, transferring, sorting and storage of both household hazardous materials and hazardous waste from businesses.

The flooring in the work area, does not meet the requirements of Chapter 123 (provided below for reference):

IAC 567-123.8(6) All receiving, sorting, bulking, transfer and storage area surfaces shall be constructed of an impervious, smooth material so designed to be easily cleaned, nonreactive with the waste, and with proper drainage, in the form of plastic-lined pits or concrete sumps, according to applicable codes. Areas used for the receiving, bulking, transferring, lab packing and storing of exempt hazardous materials shall be provided with secondary containment and shall be protected from exposure to the weather.

Dubuque has previously been notified of this requirement in writing in on 20th of February 2014. Since that notification based upon Semi Annual reports provided to the DNR, and RCC staff discussions, operations have continued for approximately 2 years.

CONCLUSIONS AND RECOMMENDATIONS:

- No RCC operations may take place in the building in its current condition.

  - The RCC is currently closed for the winter.

- Any materials on the surface need to be moved immediately:

- Collected by a Licensed Haz Waste Contractor or

- Moved to the Haz Storage Building

- Batteries are on Secondary Containment

  - Once the container is cleaned of free standing liquid,

Batteries maybe collected on the newly poured concrete in the NW corner of the building.

It is required that this building be brought into compliance with IAC 567-123.8(6) prior to resuming operations within the building this coming Spring.  Questions regarding the technical aspects of meeting this requirement should be addressed to Kathleen Hennings at Kathleen.Hennings@dnr.iowa.gov or by phone at 515-725-8359.

It is further required that a plan of action to bring this building into compliance be developed and submitted to this office within the next 60 days.