[image: image1.png]





IOWA DEPARTMENT OF NATURAL RESOURCES

SINGLE USE LANDFARM (PCS) PERMIT INSPECTION FORM
	Permit No.:77-SDP-60-07P-PCS
	County:Dubuque

	Facility Name:Portzen Construction, Inc. 

Phone Number:563/557-7642
	Facility Address:

205 Stone Valley Drive
Dubuque, IA 52003


	Responsible Official:Mike Portzen Jr.
Phone Number:563/557-7642
Person(s) Present:

1)Mike Portzen Jr.
2)Jarad Prescott (E-Farm)
3)
	Mailing Address:

Same as Above



	Date of This Inspection:08/31/09
	Date of Last Inspection:NA


*Be advised your facility may require, due to either SIC  code or  onsite management practices, an NPDES General Permit #1 (Stormwater permit).
	PCS Analysis
	IAC 567 Chapter 120.6:  Petroleum Contaminated Soil (PCS) Analysis
	Yes
	No
	NA

	
	120.6(2)“c”
	Was the PCS analyzed for the following constituents:

· BTEX Testing – benzene, toluene, ethylbenzene, and xylene (OA-1)
· TEH-diesel Testing – total extractable hydrocarbons (OA-2)
· MTBE Testing – methyl tertiary-butyl ether must be analyzed for unless prior analysis has shown MTBE is not present in soil or groundwater
· Total RCRA Metals Testing – Resource Conservation and Recovery Act Metals must be analyzed for if the history of the petroleum contaminated site included solvents, batteries, leaded fuel, waste oil, or a gas station in operation prior to 1985 
	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 



Comments:
	Site Suitability Requirements
	IAC 567 Chapter 120.7:  Site Exploration and Suitability Requirements
	Yes
	No
	NA

	
	120.7(1)
	Does the single use landfarm applicator have written confirmation from the site owner for one of the following:

· That any other landfarm created in the past three years within 15 feet of the proposed operating area has been closed pursuant to rule 567 – 120.12(455B)

· That no area within 15 feet of the proposed operating area has been used as a landfarm in the past three years

(Note: This rule does not apply for a landfarm utilized prior to October 20, 2004)
	 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(2)
	Has PCS been stored or landfarmed within 500 feet from a well (including agricultural drainage wells)?  (Note: Groundwater monitoring wells installed pursuant to paragraph 120.8(2)“c” and extraction wells as part as a  remediation system are exempt from this requirement)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(3)
	Has PCS been stored or landfarmed within 500 feet of a sinkhole?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(5)
	Has PCS been stored or landfarmed within 200 feet of a stream, lake, pond, wetland, or other surface water of the state?  (Note: The setback requirement may be waived if the surface water has been constructed for pollution control purposes)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 



	Site Suitability Requirements
	IAC 567 Chapter 120.7:  Site Exploration and Suitability Requirements (Cont’d)
	Yes
	No
	NA

	
	120.7(6)
	Has PCS been stored or landfarmed within 200 feet of a tile line surface intake?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(7)
	Has PCS been stored or landfarmed within 200 feet of an occupied residence, recreational area, child care facility, educational facility, or health care facility?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(8)
	Has PCS been stored or landfarmed within the 100 year flood plain?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(9)
	Has PCS been stored or landfarmed on slopes greater than 5 percent?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(10)“a”(2)
	Do the soils in the operating area of the single use landfarm consists one or more of the following:

· Clay, sandy clay, sandy clay loam, sandy loam, silty clay, silt clay loam, clay loam, loam, or silt loam as classified by USDA Textural Classification Chart for soils
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(10)“b”
	Are soils in the operating area free of stones and debris larger than 4 inches in diameter?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(10)“c”
	Do soils in the operating area have a pH greater than or equal to 6 and less than or equal to 9?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.7(10)“d”
	Does the landfarm operating area have a minimum of 6 feet of soil over bedrock?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 



Comments:Mike Portson stated the area had never been utilized for landfarming PCS.

Portzen Construction excavated soil to 7 feet to ensure shallow bedrock was not present.

	Landfarm Design 
	IAC 567 Chapter 120.8(1):  Landfarm Design Requirements
	Yes
	No
	NA

	
	120.8(1)“a”
	Have PCS storage areas (if any) been constructed in compliance with the following requirements:
· Over an impervious surface (e.g., tarp, concrete pad, plastic sheeting)

· Under a roof or tarp to minimize the infiltration of precipitation

· In an area with minimal potential for stormwater run-on
	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.8(1)“b”
	Have the landfarming plots being utilized been flagged pursuant to the following requirement:
· Landfarming plots being utilized shall be flagged for one year after land application or until landfarm is closed, whichever is shorter
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 



Comments:     
	Landfarm Operating Requirements
	IAC 567 Chapter 120.9:  Landfarm Operating Requirements
	Yes
	No
	NA

	
	120.9(1)
	Has only “Standard PCS” been landfarmed (unless a permit amendment is issued) on the plot being utilized as a single use landfarm?  (Note:  Standard PCS – soil contaminated with gasoline, diesel fuel, kerosene, jet fuel, motor oil, hydraulic fluid, or some combination thereof)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(2)
	Has PCS that is in a saturated, slurry, or in a flammable condition been landfarmed at the single use landfarm?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(3)
	Was PCS stored (if applicable) for seven days or less during landfarm season (April 1 – October 31)?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(4)
	If PCS was landfarmed outside of landfarming season, was it part of an emergency cleanup supervised by the department or did the following conditions exist:

· The operating area is free of snow

· The slope of the operating area is less than 3 percent

· The PCS is incorporated into the soil as soon as site conditions allow
	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(5)
	When the landfarming plot was utilized, was only one application of a particular source and type landfarmed?  (Note:  A single use landfarm may apply a subsequent application of PCS to the previously utilized landfarm plot if the plot has been closed pursuant to subrule 120.12(2).)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(6)
	Did the PCS application rate comply with the following table?  

The average concentration of benzene shall be calculated from all soil borings test results within the PCS excavation area.

Table 1

Average concentration of benzene (mg/kg)
Ft2 of landfarm area per  yd3 of PCS applied
Maximum thickness of unincorporated PCS
Yd3 of PCS per acre of landfarm
0 < mg/kg < 10
81 ft2
4 inches

537 yd3
10 < mg/kg < 20
162 ft2
2 inches

268 yd3
20 < mg/kg
324 ft2
1 inch

134 yd3
(Note:  PCS from an emergency cleanup supervised by the department may be land applied at a rate of 2 inches thick or at a rate of 162 ft2 of landfarm area per cubic yard of PCS.  PCS that is not contaminated with benzene or MTBE, but is contaminated with toluene, ethylbenzene, xylene, TEH-diesel, or some combination thereof, shall be land applied at a rate of 81 ft2 of landfarm area per cubic yard of PCS)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(8)
	Has solid waste that is not PCS (e.g., pipe) been removed and properly disposed of?  (Note:  This includes all rubble, stones, debris larger than 4 inches in diameter, or anything that interferes with incorporating and turning of the PCS)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(9)
	Was the PCS incorporated into the soil by tilling, disking, or other suitable means within 48 hours after it was land applied or before a precipitation event, whichever was sooner?  (Note:  PCS must not be incorporated deeper than 12 inches)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(10)
	After incorporation, was the PCS turned by tilling, disking, or other suitable means at least once per month for the first three months during landfarming season?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.9(11)
	Were crops for human or livestock consumption grown within 15 feet from the utilized landfarm plot?  (Note:  Crops for human and livestock consumption may be grown at a single use landfarm after  the landfarm plot is no longer required to be flagged pursuant to subrule 120.9(7).)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 



	Landfarm Operating Requirements
	IAC 567 Chapter 120.9:  Landfarm Operating Requirements (Cont’d)
	Yes
	No
	NA

	
	120.9(13)
	Has PCS been removed from a single use landfarm before the landfarm was closed or the following conditions were met:
· One sample from each 2,500 ft2 (e.g., 50 foot x 50 foot area) of landfarm plot is analyzed pursuant to subparagraphs 120.6(2) “c” (1), (2), (3).  A minimum of one sample per landfarm plot shall be obtained.  All samples shall be obtained from between the top 2 to 6 inches of soil.
· The results of the tests in paragraph 120.12(1) “a” demonstrate that petroleum constituent concentrations for benzene, toluene, ethylbenzene, TEH-diesel, and MTBE are below the detection limits required by 567  - Chapter 135.
· Records of the lab results, amount of PCS removed, and the exact final location of the PCS shall be maintained by the landfarm.
	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 



Comments:Rubble greater than four inches in diameter was present in the PCS and was not removed.  Disking/tilling had not been completed when I was onsite as they were in the process of finishing up the land application.
	Reporting & Record-Keeping Requirements
	IAC 567 Chapter 120.10 and 120.11:  ERRAP Requirements
	Yes
	No
	NA

	
	120.10(1)
	Is an Emergency Response and Remedial Action Plan (ERRAP) readily available?  (Note:  Single use landfarm applicators shall have employees carry a copy of the ERRAP document to each site where operations are taking place)
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.10(3)
	At a minimum, have all employees received annual training sufficient to understand and utilize ERRAP documents?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.11(1) ”a” and “b”
	Has a complete Landfarming and Storage Notification Form(s) been submitted to the department central office as well as the department field office with jurisdiction over the landfarm before receipt of PCS for storage and land application events?
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 


	
	120.11(2)
	Has the single use landfarm agency maintained records of all information related to compliance with this chapter and the permit throughout the life of the landfarm and for three years after landfarm closure?
· Permit application information
· PCS analysis and characterization
· Site suitability information
· Specific design requirements
· Operation information
· ERRAP documents
· Reports that were submitted to IDNR
· Closure information
	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 


 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 


 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 



Comments:An ERRAP was not available during my site visit.
	Landfarm Closure
	IAC 567 Chapter 120.12:  Single Use Landfarm Closure
	Yes
	No
	NA

	
	120.12(2)
	If a landfarm is considered closed by the landfarming agency, has it been 3 years after the PCS was land applied or at least 6 months and the following was documented:

· One sample from each 10,000 ft2 (e.g., 100 x 100 foot area) of a landfarm plot is analyzed pursuant to subparagraphs 120.6(2) “c” (1), (2), and (3) (BTEX, TEH-diesel, MTBE).  A minimum of one sample per landfarm plot shall be obtained.  All samples shall be obtained from between the top 2 to 6 inches of soil.
· The results from the previous paragraph demonstrate that petroleum constituent concentrations are less then 0.54 mg/kg for benzene, 42 mg/kg for toluene, 15 mg/kg for ethylbenzene, 3800 mg/kg for TEH-diesel and 0.02 mg/kg for MTBE.
	 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 

 FORMCHECKBOX 

	 FORMCHECKBOX 



Comments:Landfarm has not been closed.
	Summary of Requirements:
	Compliance Date:

	1)     
	     

	2)     
	     

	3)     
	     

	4)     
	     

	5)     
	     


	Summary of Reminders:

	1)Solid waste other than PCS larger than 4 inches in diameter must be removed prior to application

	2)Single use landfarm applicators must carry a copy of the ERRAP to each site

	3)     

	4)     

	5)     


	Summary of Recommendations:

	1)     

	2)     

	3)     

	4)     

	5)     


	Inspector:     
Date:     

	Reviewer:     
Date:      


Facility Photographs (if applicable)

	
	

	
	

	
	


IDNR SINGLE USE LANDFARM (PCS) PERMIT INSPECTION

Page 1 of 6

IAC 567 Chapter 120. Iowa Code Chapters 455B & 455D.


Revised 12/3/2007
IDNR SINGLE USE LANDFARM (PCS) PERMIT INSPECTION

Page 3 of 6

IAC 567 Chapter 120. Iowa Code Chapters 455B & 455D.


Revised 12/3/2007

