Friday - 9/27/24 - 10:45 a.m. - 1:00 p.m.- Annual Inspection Typed Notes

Matt Nieswender Field Notes from Internal Bertch Campus Inspection

Annual Inspection Typed Notes - Field notes also taken and scanned.

Weather: sunny and clear (dry)

NOTE: IDNR Inspection with IDNR staff was on 9/24/24

Summary of Improvement Ideas:

- Improve housekeeping around dumpsters, those loading can monitor for “spills”, especially dust and ash dumpsters.

- Improve housekeeping around dust management systems

  - Adding more rubber boots for open top dumpsters collecting dust and shavings.

  - Adding tin around Finish Mill trailer collection with new wood (Matt N knowledge).

  - Adding tin and metal support struts around the one dumpster by the Finish Mill which has previously caused concerns regarding blowing dust.  Tin will require removal of temporary tarps. (Matt N knowledge)

- Check spill kits – locate them on a map, update map (make sure they have al the contents.

- Improve education/signs to reduce cigarette litter near Rough Mill (south side) and Panel Mill (back parking).

- Investigate heavy auto leak staining at 3-4 parking spots near Rough Mill (south side break area).

- Improvement of containment of resin drips around resin tank; (drips can be reduced with drip pans, training, etc.).

- Improve erosion (add rock, dirt, splash guards) to control erosion at downspouts near Paragon and the Center.  Also keep an eye on scalping caused by mowers on slopes west of Bath.

Compliance Tasks remaining for 2024:

- Get a stormwater sample this year at next storm event.

- Make changes to plan as recommended by IDNR inspector.

- Complete training for all employees in 2024.