IOWA DEPARTMENT OF NATURAL RESOURCES

ENVIRONMENTAL SERVICES DIVISION

FIELD OFFICE 5, DES MOINES
AFO SITE SURVEY REPORT

Facility Name: Kevin Blood Dairy Farm 
ID #: 60101  



Facility Owner: Kevin Blood
Location: 1131 -A 245th St., State Center, IA 50247, Section 17, State Center Township, Marshall County, Iowa

Animal Species: Dairy Cows  

Expansion?  FORMDROPDOWN 
 - If yes, previous construction date(s): 2008
AUC: 3,150 and/or AWC: 3,150,000 lbs
MMP - Approval Date: 4/23/09 or Disapproval Date:      
FO DER Review Complete?  FORMDROPDOWN 
          Committee Review Necessary?  FORMDROPDOWN 

Investigation Date: 4/15/09
Kevin Blood (owner)Persons Contacted: 
	TABLE  FORMDROPDOWN 

STRUCTURE SEPARATION DISTANCE REQUIREMENTS AND OBSERVATIONS (ft)

	Type of Structure

 (liquid, semi-liquid, 
 and dry manure storage)

 FORMDROPDOWN 

 (confinement buildings
 include covered formed

 manure storage structures)             
	Residences1, Churches, Schools, Businesses, Unincorporated & Incorporated Areas

Uninc. - Inc.
	Public Use Areas
	Major Water Sources2, Surface Intake, wellhead or cistern of Ag Drainage Wells, known Sinkholes
	Water Sources2
	Designated Wetlands
	Right-of-way of a

thoroughfare

maintained

by a political

subdivision

	REQUIREMENTS
	 FORMDROPDOWN 
 -  FORMDROPDOWN 

	 FORMDROPDOWN 

	1000
	500
	2500
	100

	OBSERVATIONS
	>1980 - >1875
	>1875
	>1000
	~522
	>2500
	>900


1 Residences not owned by the owner of the animal feeding operation.
2 Excluding farm ponds, privately owned lakes, or when a secondary containment barrier (SCB) is provided.
	ALL ANIMAL FEEDING OPERATIONS

WELL SEPARATION DISTANCE REQUIREMENTS (REQ.) AND OBSERVATIONS (OBS.) (ft)

	Type of Structure
	Public well REQ.
	Public well OBS.
	Private well REQ.
	Private well OBS.

	
	Shallow
	Deep
	Shallow
	Deep
	Shallow
	Deep
	Shallow
	Deep

	 FORMDROPDOWN 

	1000
	400
	>1000
	>400
	400
	400
	~831
	     

	 FORMDROPDOWN 

	200
	100
	n/a
	n/a
	200
	100
	n/a
	n/a


* If no wells are observed at the time of the site survey, please be aware that any newly installed well would also have to meet the separation distance requirements listed on page one of this report.

This proposed facility as submitted by the  FORMDROPDOWN 
  FORMDROPDOWN 
 with the siting requirements of Chapter 459 of the Code of Iowa and Chapter 567 IAC 65.  On-site observations and/or measurements  FORMDROPDOWN 
 compliance with the siting requirements for this proposed facility.
GPS Coordinates of Manure Storage Structure:
Northing - 4650221, Easting - 0483105, at SW corner, within 7m, NAD 83 Zone 15
SPECIALIST:
DATE: 4/20/09    REVIEWER:  
DATE:  4/20/09

 FORMDROPDOWN 

 FORMDROPDOWN 

THIS REPORT IS FINAL AND HAS NOT BEEN ALTERED ELECTRONICAL
DATE EMAILED TO CENTRAL OFFICE: 4/23/09

