November 9, 2015

Norm Cleveringa

Norm Cleveringa Feedlot

2642 Elmwood Avenue

Rock Valley, IA 51247

SUBJECT:  Open Feedlot Compliance Inspection – Lyon County – Facility #58902

NPDES Permit #6058902

Dear Mr. Cleveringa:

Attached is a copy of the report resulting from the National Pollutant Discharge Elimination System (NPDES) compliance inspection on October 22, 2015.  Your attention is directed to the requirements and recommendations portion of the report.

If you have any questions, or feel this report does not represent the conditions at your facility, please call me at 712/262-4177.

Sincerely,

Jennifer Christian, Environmental Specialist

Jennifer.Christian@dnr.iowa.gov

Field Services and Compliance Bureau

JC:lw

c:	-Reza Khosravi, AFO, WQB, ESD, DNR, Des Moines

enc:	-NPDES Inspection Report

-NMP Inspection Form

-Photos of Site and Aerial Photo

-AFO Regulatory Status Form

IOWA DEPARTMENT OF NATURAL RESOURCES NPDES COMPLIANCE INSPECTION REPORT

NPDES Permit #6058902	Facility ID#58902	Page 1 of 4

| FACILITY | Facility: Norm Cleveringa Feedlot |  |  |  | Permit Expiration Date: 08/13/2018 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Address: 2642 Elmwood Avenue |  |  | City: Rock Valley |  |  |  |  | State: IA | Zip: 51247 |
| PERSONS INTERVIEWED | Name: Norm Cleveringa |  |  |  |  | Phone: 712-470-2718 |  |  |  |  |
| RECEIVING WATERCOURSE | Stream Name: Lizard Creek |  |  |  |  |  |  |  |  |  |
| INSPECTION DATE | Date This Inspection: October 22, 2015 | Date Last Inspection: April 6, 2010 |  |  |  |  |  | Time of Inspection: 12:30 pm |  |  |
| SYSTEM TYPE | System 1 (once per year) System 3 (April/July/Oct.) System 5 (April/May/Oct./Nov.) System 2 (July/Oct.) System 4 (Each Significant Precip. Event) AT System |  |  |  |  |  |  |  |  |  |
| ANIMAL INFORMATION | Animal Type(s) |  | Permitted Head |  |  |  | Current Head |  |  |  |
|  | Immature Dairy Cattle- Open Lot |  | 600 |  |  |  | 267 |  |  |  |
|  | Mature Dairy Cows- Open Lot |  | 70 |  |  |  | 67 |  |  |  |
|  | Immature Dairy Cattle- Confinement |  | 180 |  |  |  | 130 |  |  |  |
|  | Finishing Swine- Confinement |  | 4200 |  |  |  | 4200 |  |  |  |

| PERMIT COMPLIANCE SUMMARY |  |  |  |
| --- | --- | --- | --- |
| SELF-MONITORING | Quarterly Operation Reports Submitted: Satisfactory Unsatisfactory* | Required Data on Quarterly Report: Satisfactory Unsatisfactory* | Routine Visual Inspections: Satisfactory Unsatisfactory* |
|  | Annual Reports Submitted: Satisfactory Unsatisfactory* | Required Data on Annual Report: Satisfactory Unsatisfactory* | Annual Manure Sample Taken: Satisfactory Unsatisfactory* |
|  | NMP Public Notice: Satisfactory Unsatisfactory* | Current NMP: Satisfactory Unsatisfactory* | Land Application Records: Satisfactory Unsatisfactory* |
|  | Animal Inventory Records: Satisfactory Unsatisfactory* | Equipment Inspection Documented: Satisfactory Unsatisfactory* | Depth Gauge Install & Monitoring: Satisfactory Unsatisfactory* |
| OBSERVATIONS | Solids Settling Structures: Satisfactory Unsatisfactory* | Basin or AT Structures: Satisfactory Unsatisfactory* | Evidence of Overflow or Discharge: Yes No |
|  | Manure Stockpiling: Satisfactory Unsatisfactory* | Management of Dead Animals: Satisfactory Unsatisfactory* | Management of Chemicals: Satisfactory Unsatisfactory* |
|  | Surface Drainage is Diverted from Openlot Satisfactory Unsatisfactory* | Animals have no contact with Waters of the U.S.: Satisfactory Unsatisfactory* | Feedstock Properly Handled & Stored: Satisfactory Unsatisfactory* |

* See comments

| AUTHENTICATION | Inspector: Jennifer Christian | Date: 11/9/15 | Reviewer: Cindy Martens | Date: 11/9/15 |
| --- | --- | --- | --- | --- |

Facility Name:  Norm Cleveringa Feedlot	Page 2 of 4

Inspection Date:  10/22/15

GENERAL FACILITY DESCRIPTION

Your operation is permitted to house 600 head of immature dairy cattle and 70 head of mature dairy cattle in open lot pens which utilize a SOFEB (Settled Open Feedlot Effluent Basin) located to the southeast.  You also house 180 head of immature dairy cattle in one confinement bedded barn and 4200 head of finishing hogs in two confinement barns with concrete manure pits beneath each hog building.

The SOFEB is designed under a System 2 which requires that you pump effluent during the months of July and October, at a minimum.

Your facility was issued an NPDES (National Pollutant Discharge Ellimination System) permit on August 14, 2013, and will expire on August 13, 2018.

Conditions were dry during the inspection and your facility was not discharging.

FACILITY EVALUATION

Pens and SOFEB

Your SOFEB is located to the southeast of your pens and was designed to contain 1,727,565 gallons of effluent.  Runoff from your east pens gravity drains towards the concrete solids settling area and into your SOFEB.  Runoff from your west pen settles in the southeast corner which is bermed up to capture the drainage.  According to your engineered design and your construction permit, this effluent must be pumped to the SOFEB or land applied.  During the inspection I did not observe any pumps in this area nor was there any liquid due to dry conditions.  However, it appeared as though any overflow would travel to the south and could potentially discharge into Lizard Creek during a heavy precipitation event.  You stated that the effluent pools in the corner and slowly disappears by evaporation or absorbing into the ground.  Since this is not considered a SOFEB it cannot be used to store effluent for more than seven days.  You must comply with the conditions of your permit and pump the effluent to your SOFEB or land apply.

A small horse pen is located to the southwest of your east pens, adjacent to Lizard Creek.  This pen is not designed to drain into the SOFEB and could potentially discharge into the creek.  At the time of the inspection there were no horses in the pen and it contained some vegetation.  I explained that an AFO (Animal Feeding Operation) is a lot or facility where animals have been confined for 45 days or more in any 12 month period AND crops, vegetation, forage growth or post-harvest residues are not sustained over any portion of the lot or factiliy.

During the inspection construction was taking place on an open front barn with concrete pens.  This structure is located to the north of your west pen.  There will be no increase in animals and any runoff will drain to the east/southeast and will be captured by the SOFEB.

During the inspection there was no effluent in your SOFEB as you have not received much measurable precipitation.  Your basin was completely dry and the entire bottom of the basin was vegitated with weeds.  I explained that the roots of the vegetation may grow down through the clay liner of the SOFEB and may potentially cause your basin to leak.  Without liquids in your basin, your liner may also develop cracks which may also cause it to leak.  It is in your best interest to mow and remove the cut vegetation as soon as possible and to always maintain enough liquid in your basin to cover the bottom.

Facility Name:  Norm Cleveringa Feedlot						Page 3 of 4

Inspection Date:  10/22/15

Please keep in mind that you must operate your facility according to your permit or you will not be protected by your permit to discharge during a 25-year, 24-hour precipitation event.

Manure Stockpiling

Dry manure is stockpiled on site to the northwest of your east pens.  Drainage is directed towards your SOFEB and appeared to be in a good location.  As a reminder, you must comply with the following stockpiling regulations found in the Iowa Administrative Code:

•  the stockpile is not in a grassed waterway or where water pools or has concentrated flow,

•  the stockpile is on land with less than 3 percent slope or has adequate methods, structures or practices in place to contain stockpile solids for land with greater than 3 percent slope, and

•  the stockpile complies with required separation distances to sensitive areas. Please note that the stockpile must be land applied within six months after creation and in a manner that does not cause surface or groundwater pollution.

Feed Storage Area

All of your feed is stored across the road to the west at Winding Meadows Dairy since you custom feed for the owner,Terry Van Maanen.  All of the silage is covered and contained in bunkers at the dairy and there are no runoff issues.

Bales were intact and did not appear to be inadequate for use at your feedlot.  Be aware that feed stocks have the potential to cause water quality violations and these areas should be monitored regularly.   Paragraph 567 IAC 61.3(2) outlines general surface water quality criteria for the State of Iowa and requires, among others, that all surface waters be free from materials attributable to wastewater discharges or agricultural practices producing objectionable color, odor or other aesthetically objectionable conditions.

Confinements

You have 4200 head of finishing hogs in two confinement barns located on the west side of your facitliy.  The liquid hog manure is stored in deep concrete pits beneath each confinement building.  During the inspection, I observed the manure storage structures to be structurally sound and to adequately contain manure with no evidence of manure discharges.  No leakage of manure was observed and the buildings appeared to be well maintained.  No manure was observed around load out areas.  It is recommended that you inspect for discharges and needed repairs regularly, as confinement feeding operations must contain all manure produced between periods of application.

Feed for your hog confinement operation is contained in a bulk bin feed system.  The bins, feed tubes and various attachments appeared to be intact and effectively containing feed.  There was no feed spilled on the ground from transfer of feed from the transport vehicle to the bins.  It is encouraged that this part of the operation be inspected on a frequent basis as any feed spilled could be carried away with storm water to the receiving water source where it would have the potential to create water quality violations.

You also have 180 head of immature dairy cattle in one bedding pack confinement hoop barn located south of your west pen.  This barn utilizes corn stalks as bedding.  It is cleaned out as needed and manure is hauled to the stockpiling storage area.  During the inspection I observed some dry manure along the exterior of the building near the east entrance.  You must keep this area clean of manure since it does not drain into the SOFEB.

Facility Name:  Norm Cleveringa Feedlot						Page 4 of 4

Inspection Date:  10/22/15

SELF MONITORING

Operation Reports

You have done a satisfactory job of submitting your quarterly and annual reports in a complete and timely manner.

Monitoring and Record Keeping

You have been keeping organized records of your monitoring results listed in the NPDES permit.  The spreadsheets and forms that are utilized assist in recording the required information.

Nutrient Management Plan

The NMP for your facility has been approved.  Your annual update for your confinement is due every June 1.  Solid manure is surface applied according to the rates listed in the NMP.  Liquid open lot manure is land applied by means an irrigation gun at a rate listed in the NMP.  Liquid hog manure is knifed into the soil.  You land apply all of your manure from your facility and you are a certified confinement site manure applicator, #7506.

Miscellaneous

Rendering appears to occur on time.

There are no wells on site.  Rural water is utilized at your facility.

REQUIREMENTS AND RECOMMENDATIONS

•	If your small horse pen meets the definition of an AFO, you cannot discharge into Lizard 	Creek.

•	Mow the weeds in your SOFEB and remove cuttings.  Always maintain enough liquid to 	cover the bottom of your basin.

•	Your facility must pump liquids from the SOFEBs down to 10 percent of the system’s 	design storage volume during the application periods.

•	Keep all manure application records for at least five years.

• 	Operate your facility according to the NPDES permit requirements.

•	Update the NMP as needed.

•	Report any manure spills to the Environmental Emergency Response number during 	non-business hours.  This phone number has changed to 515-725-8694.