	IOWA DEPARTMENT OF NATURAL RESOURCES

Field Office 6

1023 West Madison, Washington, IA 52353

319-653-2135

Closed Sanitary Landfill Visit

	Permit #:  90-SDP-9-93C                            Date this Visit: 9/30/16                Previous Inspection Date: 10/6/14
Weather Conditions at Time of Visit:  75° F, Sunny
Person(s) Contacted: Steve Jones


	Were deficiencies noted or significant observations made during this inspection?

Yes – See Comments Sec., No – No deficiencies noted, Blank – Not applicable or observed, PND – Previously Noted Deficiency

Observations

	Yes
	No
	PND
	Item
	Yes
	No
	PND
	Item

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	1. Fill Cover: daily; intermediate; final
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	10. Special Waste Handling 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	2. Control of Face: slope; width; location
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	11. Recycling Activities

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	3. Compaction
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	12. Operator Certification

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	4. Cover erosion; Ponding
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	13. Staffing

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	5. Drainage into fill
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	14. Equipment Backup/Maintenance

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	6. Leachate Mgmt. & Control
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	15. Interior Roads

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	7. Litter
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	16. Perimeter fence, gate, signs, mon. wells

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	8. Wet Weather Area 
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	
	17. Photos taken

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	 FORMCHECKBOX 

	9. Construction & Demolition Area
	 FORMCHECKBOX 

	 FORMCHECKBOX 

	
	18. Follow-up needed

	Location of Work Face: N/A
Comments:  On September 30, 2016 I met with Ottumwa Landfill Manager Steve Jones to conduct an inspection of the landfills closed portion.  

Vegetation over the entire closed area was thick and well established.  It also appeared to have been mowed earlier in the summer which eliminated any woody vegetation and made observation of the landfill cap possible.    
The area along the southeast boundary where the creek bed has eroded the landfill cap and exposed waste has been cleared and mowed but has yet to be repaired.  Some trees along the creek bed are planned to be removed and rip rap put in place for stabilization.  Mr. Jones indicated that the contractor plans to complete the work yet this fall.  While doing this work the contractor also plans to establish correct cover on the portion of the recently discovered landfill in the northeast corner of the closed cell.  This area, which was discovered from historic aerial photographs, was recently purchased from the adjacent landowner and will be provided with appropriate cover and re-vegetated.  Due to its remote location it has been determined that perimeter fence will not be required.  In addition to these tasks the contractor will also haul a stockpile of dirt to the working cell(s) to be used as cover for the following year.  

No issues were determined with the monitoring wells.   
We also briefly observed the MSW working face.  It appeared as if adequate cover had been applied to the previous days fill area and minimal on and off site litter was observed.  


	AUTHENTICATION

	INSPECTOR: Bert Noll


	REVIEWER: Kurt Levetzow




