Kraft Heinz Mason City - Facility No. 017-01-056

Retification of Deficiencies: EP-SU9 Case Code Ink Printing

Records of case code ink usage is kept on spreadsheet electronically by the Materials Coordinator. The ink currently used at the plant is Little David Jet Printer Ink CPA275-BK which contains a glycol ether ingredient, dipropylene glycol monomethyl either. The information from the SDS needed for this emission calculation is the specific gravity of the product (0.92), amount of ingredient in the product composition (3.92%), and 2022 usage (1980 gal). That information inserted into the VOC Emission calculation comes to .30 tons of VOC emissions. The Case Code Ink SDS will be submitted along with this document.

1. 2022 Case Code Ink Usage: