November 24, 2015

Mike Rozema

1741 Vine Avenue

Ocheyedan, IA 51354

SUBJECT:  Animal Feeding Operation (AFO) Compliance Inspection for – Mike Rozema, County – Osceola, – Facility #61052

Dear Mr. Rozema:

Attached is a copy of the report resulting from the Animal Feeding Operation (AFO) facility compliance inspection on November 12, 2015.

Your attention is directed to the requirements and recommendations portion of the report.

If you have any questions, or feel this report does not represent the conditions at your facility, please call me at 712-262-4177.

Sincerely,

Jennifer Christian, Environmental Specialist

Jennifer.Christian@dnr.iowa.gov

Field Services and Compliance Bureau

c:	-Gene Tinker, AFO Coordinator, Des Moines

-Ken Hessenius, FO#3, AFO Enforcement Coordinator, Spencer, IA

enc:	-AFO Facility Inspection Report

-MMP Inspection Form

-Earthen Manure Storage Structure Inspection Form

-Animal Feeding Operation (AFO) Regulatory Status Form

-Desktop Assessment Form

-Photos of Site and Aerial Photo

Page 1 of 3

IOWA DEPARTMENT OF NATURAL RESOURCES AFO INSPECTION REPORT

| FACILITY DESCRIPTION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY LOCATION | Facility: Mike Rozema |  |  |  |  | Facility ID#: 61052 |  |
|  | Address: 1741 Vine Avenue |  | City: Ocheyedan |  | State: IA |  | Zip: 51354 |
|  | PLSS: Section 18, Allison Township (T99N, R39W), Osceola County |  |  |  |  |  |  |
| OWNER | Name: Mike Rozema |  |  |  |  |  |  |
| ANIMAL HOUSING TYPE | Confinement Open Lot Combined (Confinement & Open Lot) |  |  |  |  |  |  |
| ANIMAL INFORMATION | Animal Type(s) | Capacity |  | Current Head | Number of Bldgs./Pens |  |  |
|  | Swine Finisher | 3720 |  | 600 | 3 |  |  |
|  | Date of Construction: 1994 |  |  | Date of Expansion: 1998 |  |  |  |
| INSPECTION INFORMATION |  |  |  |  |  |  |  |
| INSPECTION DATE | This Inspection: November 12, 2015 |  |  | Last Inspection: August 6, 2007 |  |  |  |
| PERSONS INTERVIEWED | Name: Jess Rozema |  |  | Title: Site manager |  |  |  |
| NEAREST WATERCOURSE | Stream Name: Tributary of the Ocheyedan River |  |  |  |  |  |  |
|  | Description of Flow Path: from east to west |  |  |  |  |  |  |

| COMPLIANCE SUMMARY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OBSERVATIONS | Nutrient Management: CNMP NMP MMP Other No formal plan |  |  |  |  |  |  |
|  | Manure Stockpiling: In controlled area In compliance with rules Not applicable – direct haul Stockpiling in an uncontrolled area |  |  | Mortality Management: Rendering Composting Incineration On-site burial Landfill |  | Runoff from Feed Storage: No outdoor feed storage area Discharge from feedstock storage area is controlled Feed storage is located in an uncontrolled area |  |
|  | Clean Water Diverted: Yes No |  |  | Discharge to a Water of the U.S. via Manmade Conveyance: Yes No |  | Direct Animal Contact with Waters of the U.S.: Yes No |  |
|  | Adjacent Facilities (by same owner/operator): Confinement Open Lot None |  |  |  |  |  |  |
|  | Evidence of Discharges: Yes No |  |  |  |  |  |  |
| NPDES PERMIT STATUS | The facility, as observed during the inspection, was a and an NPDES permit. NPDES permit is required: Yes No |  |  |  |  |  |  |
| COMPLIANCE STATUS | This facility to be in compliance with Iowa’s environmental regulations at the time of the inspection. Actual conditions may vary over time with the operation and maintenance of the facility. Facility is in compliance: Yes No |  |  |  |  |  |  |
| AUTHENTICATION |  | Inspector: Jennifer Christian | Date: 11/24/15 |  | Reviewer: Cindy Martens |  | Date: 11/24/15 |

Page 2 of 3

IOWA DEPARTMENT OF NATURAL RESOURCES AFO INSPECTION REPORT

Facility Evaluation

Your facility has the capacity of housing 3720 head of finishing swine in three confinement buildings.  The north barn utilizes a concrete manure storage structure beneath the confinement.  The other two confinement buildings utilize a permitted earthen manure storage structure located to the west.  Your facility is located in Section 18, Allison Township, Osceola County, Iowa.  It is classified as a Large Concentrated Animal Feeding Operation (CAFO) because it has a capacity of 2,500 or more head of swine weighing more than 55 pounds.

Confinement Buildings

During the inspection, I observed the manure storage structures to be structurally sound and to adequately contain manure with no evidence of manure discharges.  No leakage of manure was observed and the buildings appeared to be well maintained.  No manure was observed around load out areas.  It is recommended that you inspect for discharges and needed repairs regularly, as confinement feeding operations must contain all manure produced between periods of application. Any discharges may require a National Pollutant Discharge Elimination System (NPDES) permit for the operation.

Feed for your confinement operation is contained in a bulk bin feed system.  The bins, feed tubes and various attachments appeared to be intact and effectively containing feed.  There was no feed spilled on the ground from transfer of feed from the transport vehicle to the bins.  It is encouraged that this part of the operation be inspected on a frequent basis as any feed spilled could be carried away with storm water to the receiving water source where it would have the potential to create water quality violations.  In addition, a discharge of such pollutants would potentially require an NPDES permit.

Since your facility houses over 2500 head of swine weighing over 55 pounds, it is considered to be a Large CAFO (Concentrated Animal Feeding Operation).  Large CAFOs are subject to obtain a federal NPDES (National Pollutant Discharge Elimination System) permit and retain all runoff or claim to not have a discharge.  Since your hogs are housed in confinement barns you are claiming to not have a discharge of manure or contaminated wastewater to a water of the United States.

Manure Management

Your facility has an approved Manure Management Plan (MMP) on file with the IDNR Field Office #3 in Spencer, Iowa.  The MMP annual update is due on August 1st each year.  A complete, updated MMP is required to be submitted to FO#3 at least once every four years.

Your facility annually produces an estimated 1,540,080 gallons of manure and is typically land applied in the fall by commercial manure applicator, Tom Doeden, #2292.  The current MMP contains approximately 953 acres available for manure application which is sufficient considering the manure application rate, crop rotation, etc. listed in the plan.  Your facility has had no issues with storage of manure in the past and a review of manure application records showed that no winter manure application has been necessary.

Miscellaneous

All deceased animals are rendered.

There is one deep well located to the east of the south confinement barns.

Bio-Security

The approved DNR bio-security policy was followed during the inspection.

Documentation Collected

Photographs representing the site conditions were taken and printed for this report.

Page 3 of 3

Requirements/Recommendations

•	MMP annual update for the facility is due August 1st of each year; a complete, updated MMP must be 			submitted to the Iowa DNR at least once every four years.

•	Soil samples used to calculate the P-Index for fields contained in the MMP must be 4 years old or less.

•	Manure application records must be maintained for 5 years.

•	Notify the Iowa DNR of any manure release as soon as possible but not later than six hours after the onset 		or discovery of the release by contacting the department at (712)262-4177 during normal business hours, 			or at (515) 725-8694 after hours, holidays, weekends, etc.

•	As a confinement feeding operation, this facility must be maintained as a no discharge site, with all manure 		contained as well as contaminated wastewater.

Summary

This facility is classified as a Large Concentrated Animal Feeding Operation (CAFO) because it has a capacity of housing 2,500 or more head of swine weighing more than 55 pounds.  There was no evidence of discharge to any surface waters from the facility witnessed during the on-site inspection, no flow paths were observed from the confinement buildings and there have been no spills or releases from this facility documented in the past, therefore no NPDES permit is required.

As owner of the confinement, you are responsible to comply with all applicable regulations.  If you have any questions, please call our Field Office #3 at 712-262-4177.

| IOWA DEPARTMENT OF NATURAL RESOURCES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AFO Desktop Assessment Form |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Assessor: | Jennifer Christian |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Assessment Date: |  |  |  |  |  |  |  |  |  |  |  | 11/12/15 |  |  |  |
| Documentation Examined: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AFO Siting Atlas |  |  | Facility File |  |  |  |  |  |  | FOCD |  |  |  |  |  |  |  |  |  | AFO Database |  |  |  |  |  |  |  |  |  |  |  |
| MMP |  |  | Public Mapping Information |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FACILITY LOCATION |  | FACILITY: Mike Rozema |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FACILITY ID#: 61508 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ADDRESS: 1741 Vine Avenue |  |  |  |  |  |  |  |  | CITY: Ocheyedan |  |  |  |  |  |  |  | STATE: IA |  |  |  |  |  |  |  | ZIP: 51354 |  |  |  |  |  |
|  |  | PLSS: Section 18, Allison Township (T99N, R39W), Osceola County |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OWNER |  | NAME: Mike Rozema |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ANIMAL INFORMATION |  | ANIMAL TYPE(S) |  |  |  |  | CAPACITY |  |  |  |  | CURRENT HEAD |  |  |  |  | # OF PENS |  |  |  |  |  |  |  |  | # OF BUILDINGS |  |  |  |  |  |  |
|  |  | Swine Finisher |  |  |  |  | 3720 |  |  |  |  | 600 |  |  |  |  | NA |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |
| FACILITY TYPE |  | Confinement |  |  |  |  |  | Open Lot |  |  |  |  |  |  |  |  |  |  | Combined |  |  |  |  |  |  |  |  |  |  |  |  |
| STORAGE TYPE |  | Liquid |  | Dry |  |  |  |  |  |  |  |  | Covered |  |  |  |  |  |  |  |  |  |  | Uncovered |  |  |  |  |  |  |  |
| STORAGE STRUCTURE TYPE |  | Earthen Manure Storage Structure |  |  |  |  |  |  | # 1 |  |  |  | Anaerobic Lagoon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # |  |
|  |  | Below Building Pit |  |  |  |  |  |  | # 2 |  |  |  | Aerobic Lagoon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # |  |
|  |  | Outside Concrete Pit |  |  |  |  |  |  | # |  |  |  | Outside Stockpile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # |  |
|  |  | Slurry-store |  |  |  |  |  |  | # |  |  |  | Covered Stockpile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # |  |
|  |  | Unknown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AFO/CAFO Status |  | Large CAFO† |  |  | Medium AFO |  |  |  |  |  |  |  | Small AFO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | † All large CAFOs require an onsite inspection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NEAREST WATERCOURSE |  | Watercourse Name: Tributary of the Ocheyedan River |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Distance between facility and nearest watercourse: |  |  |  |  |  |  |  |  |  |  |  | < ¼ mile†† |  |  |  |  |  |  |  | > ¼ mile |  |  |  |  |  |  |  |  |  |
|  |  | Description of flow path(s) to watercourse: from east to west |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | †† All medium combined or open lot AFOs within a ¼ mile of a watercourse and that drain towards that watercourse require an onsite inspection. All medium confinement AFOs that utilize uncovered manure/litter storage and are within a ¼ mile of a watercourse and that drain towards that watercourse require an onsite inspection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COMPLIANCE HISTORY |  | Has there been a discharge to a Water of the U.S. within the last 5 years? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes††† |  |  |  |  |  |  |  |  |  | No |  |  |
|  |  | If yes, did the facility permanently remedy the cause of the discharge? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | No |  |  |  |  |  | Unknown |  |  |
|  |  | †††All medium confinement AFOs that have discharged to water of the U.S. within the last 5 years require an onsite inspection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Has there been a significant release within the last 5 years? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | No |  |  |
|  |  | If yes, did the release present a significant threat of discharge? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes* |  |  |  |  | No |  |  |  |  |  | Unknown |  |  |
|  |  | * All medium confinement AFOs that have had a significant release in the last 5 years and the release presented a significant threat of discharging to a water of the U.S. require an onsite inspection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Have there been any complaint investigations? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | No |  |  |
|  |  | If yes, describe: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Has an onsite inspection been conducted at this facility since 11/1/11? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | No |
|  |  | If yes, was the inspection functionally equivalent to facility type specific SOP (i.e., confinement, open feedlot or combined)? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes** |  |  |  |  |  | No |  |  |  |  |  | N/A |
|  |  | Inspection Date: |  |  |  | Describe: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | ** No onsite inspection is required if a functionally equivalent inspection has been performed since 11/1/11. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| RUNOFF ASSESSMENT | Is there evidence that manure, litter, or process wastewater is uncontrolled and/or unmanaged? |  |  |  | Yes | No | Unknown |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | If yes, describe: |  |  |  |  |  |  |
|  | Are there tile intakes within 100 feet of the production area? |  |  |  | Yes | No | Unknown |
|  | If yes, describe: |  |  |  |  |  |  |
|  | Does the facility utilize uncovered/uncontrolled composting areas? |  |  |  | Yes | No | Unknown |
|  | If yes, describe: |  |  |  |  |  |  |
|  | Note: If assessor answered “Yes” to any of the questions in this section, then an onsite inspection should be performed. |  |  |  |  |  |  |
|  | Assessment Notes/Comments: This facility is considered a Large CAFO claiming not to discharge into a water of the United States. |  |  |  |  |  |  |
|  | ONSITE INSPECTION REQUIRED. |  | ONSITE INSPECTION NOT REQUIRED. |  |  |  |  |
| AUTHENTICATION | INSPECTOR: Jennifer Christian | DATE: 11/24/15 |  | REVIEWER: Cindy Martens |  |  | DATE: 11/24/15 |
| Note: This assessment was based on the information available on the date of the assessment. Conditions at this facility could change. |  |  |  |  |  |  |  |

|  | Animal Feeding Operation (AFO) Regulatory Status |
| --- | --- |

| Facility Name: | Mike Rozema | Facility ID: | 61052 | County: | Osceola |
| --- | --- | --- | --- | --- | --- |

Large CAFO – Discharging – NPDES Permit Required

Large CAFO – No discharge – No NPDES Permit Required

Large CAFO – Has NPDES Permit

Medium CAFO – NPDES Permit Required

Medium AFO – No NPDES Required

Medium AFO – Has NPDES Permit

Designated CAFO – NPDES Permit Required

Small AFO – No NPDES Permit Required

This determination was made based on conditions and observations made at the time of the  on November 12, 2015.  Please note that the regulatory status of the facility can change if conditions at the facility change or are different from those documented during the .

| Inspector: |  | Date: |  |
| --- | --- | --- | --- |

Regulatory Definitions of Large CAFOs, Medium CAFOs, and Small CAFOs

These regulatory definitions are from the Code of Federal Regulations (CFR), implementing the federal Clean Water Act.

A Large CAFO confines at least the number of animals described in the table below.

A Medium CAFO falls within the size range in the table below and either:

- “(A) Pollutants are discharged into waters of the United States through a man-made ditch, flushing system, or other similar man-made device; or

- (B) Pollutants are discharged directly into waters of the United States which originate outside of and pass over, across, or through the facility or otherwise come into direct contact with the animals confined in the operation.” 40 CFR 122.23(b)(6)(ii)

If an operation is found to be a significant contributor of pollutants to waters of the United States, the permitting authority may designate a medium-sized facility as a CAFO as provided in 40 CFR 122.23(c).

A Small CAFO confines the number of animals listed in the table and has been designated as a CAFO by the permitting authority after determining that it is a significant contributor of pollutants to waters of the United States as provided in 40 CFR 122.23(c).

| Animal Sector | Size Thresholds (number of animals) |  |  |
| --- | --- | --- | --- |
|  | Large CAFOs | Medium CAFOs | Small CAFOs |
| cattle or cow/calf pairs | 1,000 or more | 300 – 999 | less than 300 |
| mature dairy cattle | 700 or more | 200 – 699 | less than 200 |
| veal calves | 1,000 or more | 300 – 999 | less than 300 |
| swine (weighing over 55 pounds) | 2,500 or more | 750 -2,499 | less than 750 |
| swine (weighing less than 55 pounds) | 10,000 or more | 3,000 – 9,999 | less than 3,000 |
| horses | 500 or more | 150 – 499 | less than 150 |
| sheep or lambs | 10,000 or more | 3,000 – 9,999 | less than 3,000 |
| turkeys | 55,000 or more | 16,500 – 54,999 | less than 16,500 |
| chickens other than laying hens (other than a liquid manure handling system) | 125,000 or more | 37,500 – 124,999 | less than 37,500 |
| laying hens (other than a liquid manure handling system) | 82,000 or more | 25,000 – 81,999 | less than 25,000 |

|  |  |  |  | IOWA DEPARTMENT OF NATURAL RESOURCES ENFORCEMENT CHECKLIST FOR AFO/CAFO INSPECTIONS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INSPECTION DESCRIPTION |  |  |  |  |  |  |  |  |  |  |
| Date of Inspection |  |  |  | 11/12/15 |  |  |  |  |  |  |
| Facility Name |  |  |  | Mike Rozema |  |  |  |  | Facility ID# | 61052 |
| Facility Address |  |  |  | 1741 Vine Avenue |  |  |  |  |  |  |
| Inspector’s Name |  |  |  | Jennifer Christian |  |  |  |  |  |  |
| INSPECTION FINDINGS |  |  |  |  |  |  |  |  |  |  |
| Narrative Description of Investigation (evidence of current violations; indicators of past violations; future concerns): |  |  |  |  |  |  |  |  |  |  |
|  | Photographs and/or Video |  |  |  |  |  |  |  |  |  |
|  | Water Samples (upstream and downstream) |  |  |  |  |  |  |  |  |  |
|  | Personal Interviews |  |  |  |  |  |  |  |  |  |
|  | Other |  |  |  |  |  |  |  |  |  |
| ACTIONS FOLLOWING INSPECTION |  |  |  |  |  |  |  |  |  |  |
|  | No further action taken – no violation(s) observed |  |  |  |  |  |  |  |  |  |
|  | Informal Meeting |  |  |  |  | Date |  |  |  |  |
|  | Letter of Inquiry |  |  |  |  | Date |  |  |  |  |
|  | Letter of Noncompliance |  |  |  |  | Date |  |  |  |  |
|  |  | (Within 30 days of confirmation of Violation) |  |  |  |  |  |  |  |  |
|  | Notice of Violation Letter |  |  |  |  | Date |  |  |  |  |
|  |  | (Within 30 days of confirmation of Violation) |  |  |  |  |  |  |  |  |
| REFERRAL/NON-REFERRAL |  |  |  |  |  |  |  |  |  |  |
|  | Non Referral; No referral warranted. Explanation: |  |  |  |  |  |  |  |  |  |
|  | Referral; based on the following criteria: |  |  |  |  |  |  |  |  |  |
|  | Fish kill/acute water quality degradation |  |  |  |  |  |  |  |  |  |
|  | (Manure spills and/or discharges that result in destruction of aquatic life, including fish, are a top priority) |  |  |  |  |  |  |  |  |  |
|  | Serious water quality degradation |  |  |  |  |  |  |  |  |  |
|  | (Release of pollutants may result in degradation of an aquatic resource without an obvious fish kill, but the effect may be impaired use and enjoyment of the water resource or chronic pollution harming aquatic life) |  |  |  |  |  |  |  |  |  |
|  | Discharges of pollutants to state waters not authorized by an NPDES permit |  |  |  |  |  |  |  |  |  |
|  | (This priority would include discharges from open feedlots or confinements to waters of the state, not authorized under conditions of an NDPES permit issued by the DNR. An impact on water quality is documented) |  |  |  |  |  |  |  |  |  |
|  | Failure to obtain required NPDES permit |  |  |  |  |  |  |  |  |  |
|  | (A large CAFO, medium CAFO, or designated CAFO is found to have any documented discharge without, or in violation, of an NPDES permit) |  |  |  |  |  |  |  |  |  |
|  | Unauthorized construction |  |  |  |  |  |  |  |  |  |
|  | (Construction of AFO/CAFO structures (including open feedlots) without, or contrary to, a permit or other required documentation is also a DNR priority. Proper compliance with AFO siting and construction requirements is essential elements of the AFO program, which helps keep pollutants out of streams) |  |  |  |  |  |  |  |  |  |
|  | Significant violations of NPDES permit and/or conditions in the permit |  |  |  |  |  |  |  |  |  |
|  | (Violations of a significant nature and/or repeated violations of operating or reporting requirements) |  |  |  |  |  |  |  |  |  |
|  | Failure to submit MMP updates |  |  |  |  |  |  |  |  |  |
|  | (MMPs are the cornerstone of the animal feeding program. The MMP helps ensure that any proposed or current confinement feeding operation over 500 animal units has adequate land to use the manure nutrients it produces) |  |  |  |  |  |  |  |  |  |
|  | Failure to obtain proper manure application certification |  |  |  |  |  |  |  |  |  |
|  | (The manure applicator certification program is an important component of the AFO regulations. The program ensures that manure is transported and applied properly) |  |  |  |  |  |  |  |  |  |
|  | Other |  |  |  |  |  |  |  |  |  |
| Date of Referral to Legal |  |  |  |  |  |  |  |  |  |  |

|  |  |  | IOWA DEPARTMENT OF NATURAL RESOURCES ENVIRONMENTAL SERVICES DIVISION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Field Office #1 909 W Main Ste 4 Manchester IA 52057 563-927-2640 |  |  |  |  | Field Office #2 2300 15th St SW Mason City IA 50401 641-424-4073 |  |  |  |  |  |  |  | Field Office #3 1900 N Grand Ave Spencer IA 51301 712-262-4177 |  |  |  |  | Field Office #4 1401 Sunnyside Ln Atlantic IA 50022 712-243-1934 |  |  |  |  | Field Office #5 7900 Hickamn Rd Ste 200 Windsor Heights IA 50324 515-725-0268 |  |  |  |  | Field Office #6 1023 W Madison Washington IA 52353 319-653-2135 |  |  |  |  |  |  |
| ###### Manure Management Plan Compliance Review – Nonsales of Manure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site Visit Date Current: 11/16/15 Last: 2/18/11 |  |  |  |  |  |  |  | Number of Animals: 3720 |  |  |  |  |  |  | Animal Units: 1488 |  |  |  |  | N-based P-based |  | Facility Location County: Osceola |  |  |  |  |  |  | Facility ID: 61052 |  |  |  |  |  |
| ## Facility Tier Status ## Pre-visit: ## Post-visit: |  |  |  |  |  |  | 1 |  | 2 | 3 | ## Violation(s) Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | ## No violations were observed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clerical Violation(s) Technical Violation(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Facility Name |  |  | ## Mike Rozema |  |  |  |  |  |  |  |  |  |  | ## Person(s) Contacted |  |  |  |  | ## Mike |  |  |  |  | ## Phone |  | ## 712-330-8540 |  |  |  |  |  |  |  |  |
| ## Facility Owner Information & Address Unchanged? Yes No (Database must be updated) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Address of Facility: |  |  |  |  |  | ## 1741 Vine Avenue, Ocheyedan IA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Do you own or operate any other animal feeding operation within 2,500 feet? Yes No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### MMP Contents Are requirements being met for the following items? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | #### Yes |  |  |  |  | #### No | #### N.A. |  |
| a. | Calculations to determine land area for application [65.17(4)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b. | Total nitrogen and phosphorus available, credits, application losses [65.17(5)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| c. | Optimum crop yield, crop usage rate, and soils maps [65.17(6)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| d. | Manure application methods and timing of application [65.17(7)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| e. | Location of manure application and landowner agreements [65.17(8)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| f. | Estimate of annual animal production and amount of manure produced [65.17(9)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| g. | Methods, structures, or practices to reduce soil loss and potential surface water pollution (conservation plans for highly erodible land) [65.17(10)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| h. | Does P Index no. generated correspond with P Index no. in plan? [65.17(17)] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i. | Are proper soil sampling methods being used to calculate P Index?[65.17(16)] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### Records Review Are requirements being met for the following items? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | #### Yes |  |  |  |  | #### No | #### N.A. |  |
| j. |  | Copy of the current MMP [65.17(12)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| k. |  | Records kept for 5* years (or crop rotation if longer) maintained [65.17(13)]? *10/25/06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| l. |  | Methods of application [65.17(13)a]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| m. |  | Dates when manure was applied [65.17(13)b]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n. |  | Location of manure application and number of acres [65.17(13)c]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| o. |  | Manure application rate [65.17(13)c]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| p. |  | Statement of Intent for fields not owned or rented [65.17(13)e] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### Field Observations of Manure Storage/Handling/Application & Animal Disposal Are requirements being met for the following items? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | #### Yes |  |  |  |  | #### No | #### N.A. |  |
| q. |  | Manure storage structures appear to be properly operated & maintained [65.2&65.15]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| r. |  | Dead animals disposed of properly [567 IAC 100.4(455B) & Code of Iowa 167.18]? Type: Compost Incinerator Burial Rendering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| #### Discussion Items: Were the following items discussed? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | #### Yes |  |  |  |  | #### No | #### N.A. |  |
| s. |  | Manure release notification made within 6 hours [65.2(9)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| t. |  | Deficiencies, violations, and tier status? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| u. |  | Recommendations, improvements, environmental issues? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| v. |  | Manure applicator is certified [65.19]? Applicator name/number: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| w. |  | Proper manure application [65.3(2)-(3)]? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| All records were complete and very organized. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPECIALIST NAME |  |  |  | Jennifer Christian |  |  |  |  |  |  |  |  |  |  |  | ON-SITE CONTACT |  |  |  |  | Mike Rozema |  |  |  | DATE |  |  |  |  | 11/12/15 |  |  |  |  |