[image: image7.jpg]‘\\\\ ( l/,,'

Fields of Opportunities

STATE OF IOWA

TERRY E. BRANSTAD, GOVERNOR DEPARTMENT OF NATURAL RESOURCES
KiM REYNOLDS, LT. GOVERNOR CHUCK GIPP, DIRECTOR




Tri City Blacktop, Fugitive Dust Complaint Investigation Memo
Page No. 3

MEMO

DATE:
May 11, 2017
TO:
Tri City Blacktop AQ File
FROM:
Kurt Levetzow, ESS, FO6
RE:
Fugitive dust complaint investigation, FOCD Complaint No. 24046
FACILITY:
Tri City Blacktop
FACILITY NO.
82-02-017 / PP-68-000
On May 10, 2017, I investigated a fugitive dust complaint by meeting with Tri City Blacktop (herein Tri City) owner Rick Carpenter.  Tri City allows contractors to dump their concrete rubble on a stockpile, which is eventually fed into a crushing operation.  The process includes: feeding concrete rubble into a primary crusher which is conveyed into a screen/secondary crusher, then the crushed material travels across two conveyors to be stockpiled as finished “gravel substitute”.  The pictures below show the operation:
[image: image1.jpg]


 [image: image2.jpg]



	Excavator breaking up large chunks of concrete
	Loader feeding primary crusher


[image: image7.jpg][image: image3.jpg]| [ % = mcontes

S



 [image: image4.jpg]i
()
'«"“!P’.‘

()
‘%
W

\;‘@
e





	Primary crusher w/ wet suppression (water spray bars) (red box highlights spray bars)
	Screen/secondary crusher with wet suppression


[image: image5.jpg]—rme——— ¢
TRy



 [image: image6.jpg]B
¥



 

	Conveyor coming off secondary grinder, equipped with spray bars
	Stockpiling conveyor, also equipped with spray bar


Tri City utilizes skid-steers with front mounted attachments used to reach under conveyors which drags out the fines which help minimize fines from piling that could lead to the generation of more fugitive dust emissions.  There is a designated water truck that waters up to three times per day, especially when conditions require (hot/dry/windy). Recordkeeping requirements associated with Tri City’s air permits was reviewed, and appeared to show compliance with respective limitations.  A follow up visit will be conducted later this summer (2017) to observe the effectiveness of the water applications on a hot/dry day.
I thoroughly explained the fugitive dust regulations to Mr. Carpenter during the walk through portion of this visit:

Iowa Administrative Code Paragraph 567 IAC 23.3(2)”c” regarding Fugitive dust - states that a person shall take reasonable precautions to prevent particulate matter from becoming airborne in quantities sufficient to cause a nuisance as defined in Iowa Code section 657.1 when the person allows, causes or permits any materials to be handled, transported or stored or a building, its appurtenances or a construction haul road to be used, constructed, altered, repaired or demolished, with the exception of farming operations or dust generated by ordinary travel on unpaved roads. Ordinary travel includes routine traffic and road maintenance activities such as scarifying, compacting, transporting road maintenance surfacing material, and scraping of the unpaved public road surface. All persons, with the above exceptions, shall take reasonable precautions to prevent the discharge of visible emissions of fugitive dusts beyond the lot line of the property on which the emissions originate.   

Overall, the operations appeared to be complying with air permit requirements, as well as general fugitive emission regulations, which basically require Tri City to take reasonable precautions to minimize fugitive emissions from crossing property lines.   
N:\FO6\SHARED\klevetz\AQ\2017\Tri City Blacktop dust complt memo 5-11-17.doc
Picture Filenames: Tri City 5-10-17.jpg
xc: AQB – DNR
Environmental Services Division, Field Office #6, 1023 W. Madison, Washington, Iowa 52353-1623 Tel 319-653-2135   Fax 319-653-2856

WALLACE STATE OFFICE BUILDING / DES MOINES, IOWA 50319

515-281-5918    TDD 515-242-5967    FAX 515-281-6794    www.iowadnr.com

